BLP MOTO

PAD125-V2-A(V4)

Assembly Manual

| - Remove the straps and cardboard cover. |  |
| --- | --- |
| - Remove all the straps and take out the parts attached on the vehicle and the packaging iron frame, remove the iron frame screws, wait for subsequent installation. - Front fender - Front number plate - Front fork assembly - Rear mud plate - Front fork guard plate - Tool box - Rear shock - Front wheel | - Make sure that all the tools in the toolbox are complete. - No.13-15 wrench - No.17-19 wrench - Spark plug box wrench - 6mm hexagon socket - 5mm hexagon socket - Screwdriver - No.14-17 wrench - User manner - Square-shaped protective vest - Tool bag - Upper and lower clamp - Front wheel axle - Self-tapping screw - Front wheel axle sleeve - Oil pipe |
| - 4.Use tool No.14-17 wrench and 6mm hexagon socket to install the handlebar. | - 5.Installed the clutch cable. |
| - 6.Use tool No.13-15 wrench to install the rear shock. | - Use tool 6mm and 5mm hexagon socket to install front fork assembly. |
| - Use tool No.14-17 wrench and No.17-19 wrench to install the front wheel (bushings are long on the left and short on the right, flange shaft enters from the left) | - Remove the fixing bolt of the caliper of the front brake, and remove the plastic buckle,and install the front brake. |
| - 10.Loosen the screws of the front guard plate, install the front guard plate, and install the front wheel axle guard sleeve. | - 11.Install the front fender |
| 12.Install the front number plate | - Install the rear mud plate |