BLP MOTO

SY70（DRONE）

Assembly Manual

| - Remove the straps and cardboard cover. |  |
| --- | --- |
| - Remove all the straps and take out the parts attached on the vehicle and the packaging iron frame, remove the iron frame screws, wait for subsequent installation. | - Make sure that all the tools in the toolbox are complete. - Tool box - Front wheel screws - Dust cover - Protective vest - Tire gauge - Tool bag - Handguard - User Manual |  |
| 4.Open the No. 7 tool bag in Step 3 - No.14-17 wrench - No.10-12 wrench - No.8-10 wrench - Spark plug sleeve - M6 hex socket - No.29 wrench | - 5.Remove the screws from the handlebar and then install it to the bracket. |  |
| - 6.Use tool No.8-10 wrench to remove the screws. Place the light in the correct position, then install the screws. | - Use tool No.14-17 wrench to remove the screws. （Both the left and right sides） |  |
| - Use tool No.29 wrench remove the nuts and washers, install the rear wheel , and then install the washers and nuts.Insert the locking pin. - The other side tire follows the same procedure. | - Use tool No.10-12 wrench and four front wheel screws(in Step3 ② Front - Wheel screws) to install the front wheel. |  |
| - Use tool No.14-17 wrench to install the front fork. | - Use tool No.8-10 wrench remove these four screws from the frame. - Install the front bumper and tighten the screws |  |
| - Open ⑦handguard in Step 3. - Insert the 1 ring onto the handle - Install the 2 handguard bracket on the handle - Fix the 3 guard hand onto the bracket using 4 screw and No.5 nut - The other side tire follows the same procedure | - Put on the protective sleeve and the protective leather. |  |
| - Install the dust cover. |  |  |