BLP MOTO

PAD90-V2(V1)

Assembly Manual

| - Remove the straps and cardboard cover. |  |
| --- | --- |
| - Remove all the straps and take out the parts attached on the vehicle and the packaging iron frame, remove the iron frame screws, wait for subsequent installation. - Front wheel - Tool box - Front number plate - Front fork assembly - Front fender | - Make sure that all the tools in the toolbox are complete. - Tool bag - Tool box - Protective vest - Upper and lower clamp - Spark plug box wrench - No.12-14 wrench - No.10-12 wrench - 6mm hexagon socket - 5mm hexagon socket - Single support - Front wheel axle - Single support spring - Bidirectional ventilation valve - Front wheel axle sleeve - User manner |

| - 4.Use tool No.10-12 wrench and 6mm hexagon socket to install the handlebar. | - 5.Use tool 6mm hexagon socket to install front fork assembly. |
| --- | --- |
| - Use tool No.12-14 wrench and No.14-17 wrench to install the front wheel (bushings are long on the left and short on the right, flange shaft enters from the left) | - Remove the fixing bolt of the caliper of the front brake, and remove the plastic buckle,and install the front brake. |

| - Loosen the screws of the oil pipe clamp, and install the front wheel axle guard sleeve. | 9.Install the front fender |
| --- | --- |
| - 10. Install the front number plate. | - 11.Install the rear shock. |
| - Use tool No.17 wrench install the single support |  |