BLP MOTO

SY125UT-D

Assembly Manual

| - Remove the straps and cardboard cover. |  |
| --- | --- |
| - Remove the tool box,make sure all the tools in the toolbox are complete. - Handguard - Tool box - Tool bag - Manual - Tire gauge - 8mm hexagon socket - 6mm hexagon socket - No.12-14 wrench - No.10-12 wrench - No.13-15 wrench - Spark plug box wrench - Screwdriver - Upper and lower clamp - The screw of the bumper - Cotter - Dust cover | - Remove the iron frame screws and take out iron frame, take out the parts attached on the vehicle. - Front shelf - Rear shelf - Rear wheel - Front wheel - Front bumper - Rear shelf bracket - Tool box |

| - Use tool No.10-12 and No.14-12 wrench to remove the iron frame. | - Install the speedometer and handlebar with tool 8mm and 6mm hexagon socket. |
| --- | --- |
| - Install the front & rear brake upper pump. | - Install the front shelf with No.10-12 wrench.(Loosen the front shelf fixing bolts,and align with the holes on the shelf and tighten the fixing screws.) |

| - Install the rear shelf with No.10-12 & No.13-15 wrench.(Loosen the fixing bolts of the rear shelf and the muffler fixing bolts,install the rear shelf bracket.) | - Remove the front bumper bolts with the No.10-12 wrench and remove the rear axle fixing bolts with the No.10-12&No.17 wrench |
| --- | --- |
| 10.Install the rear shock with the No.12-14 & No.13-15 wrench. | 11 Install the rear wheel with the No.24 wrench.(Remove the rear axle fixing bolts and install the rear wheel ,and tighten the bolts and install the big R pin and the dust cover.) |
| - Install the front fork with No.13-15 &No.17 wrench. | 13.Install the front steering track rod with No.17 wrench and small R pin |
| 14.Install the front wheel with No.13-15 wrench.(Remove the 4 fixing bolts and install the front wheel ,and tighten the bolts,and install the dust cover.) | - Install the front bumper with No.13-15 & No.10-12 wrench and 2 fixing bolts. |
| - Install the handguard. |  |